Switch Layer 3 con Vlan

Configurazione S1

Switch>enable

Switch#configure terminal

Switch(config)#hostname S1

Creazione Vlan

S1(config)#vlan 10

S1(config-vlan)#name Vlan10

S1(config-vlan)#exit

S1(config)#vlan 20

S1(config-vlan)#name Vlan20

S1(config-vlan)#exit

Assegnazione indirizzi alle vlan e attivazione interfacce virtuali

S1(config)#interface vlan 10

S1(config-if)#ip address 192.168.1.1 255.255.255.0

S1(config)#no shutdown

S1(config-if)#exit

S1(config)#interface vlan 20

S1(config-if)#ip address 192.168.2.1 255.255.255.0

S1(config)#no shutdown

S1(config-if)#exit

Attivazione routing

S1(config)#ip routing

Associazione Vlan e interfacce

S1(config)#interface fastEthernet 0/1

S1(config-if)#switchport mode access

S1(config-if)#switchport access vlan 10

S1(config)#interface fastEthernet 0/2

S1(config-if)#switchport mode access

S1(config-if)#switchport access vlan 10

S1(config)#interface fastEthernet 0/3

S1(config-if)#switchport mode access

S1(config-if)#switchport access vlan 10

S1(config)#interface fastEthernet 0/4

S1(config-if)#switchport mode access

S1(config-if)#switchport access vlan 20

S1(config)#interface fastEthernet 0/5

S1(config-if)#switchport mode access

S1(config-if)#switchport access vlan 20

S1(config)#interface fastEthernet 0/6

S1(config-if)#switchport mode access

S1(config-if)#switchport access vlan 20

S1(config)#interface fastEthernet 0/2

S1(config-if)#switchport mode access

S1(config-if)#switchport access vlan 10

S1(config)#interface fastEthernet 0/3

S1(config-if)#switchport mode access

S1(config-if)#switchport access vlan 10

S1(config)#interface gigabitEthernet 0/1

S1(config-if)#switchport trunk encapsulation dot1q

S1(config-if)#switchport mode trunk

S1(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1, changed state to up

Switch S2

Switch>enable

Switch#configure terminal

Switch(config)#hostname S1

S2(config)#vlan 20

S2(config-vlan)#name Vlan20

S2(config-vlan)#exit

S2(config)#interface vlan 20

S2(config-if)#

%LINK-5-CHANGED: Interface Vlan20, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan20, changed state to up

S2(config)#interface vlan 20

S2(config-if)#

%LINK-5-CHANGED: Interface Vlan20, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan20, changed state to up

S2(config-if)#ip address 192.168.2.1 255.255.255.0

S2(config-if)#exit

S2(config)#ip routing

S2(config)#interface fastEthernet 0/1

S2(config-if)#switchport mode access

S2(config-if)#switchport access vlan 20

S2(config-if)#exit

S2(config)#interface fastEthernet 0/2

S2(config-if)#switchport mode access

S2(config-if)#switchport access vlan 20

S2(config-if)#exit

S2(config)#interface fastEthernet 0/3

S2(config-if)#switchport mode access

S2(config-if)#switchport access vlan 20

S2(config-if)#exit

S2(config)#interface gigabitEthernet 0/1

S2(config-if)#switchport trunk encapsulation dot1q

S2(config-if)#switchport mode trunk